Bachelor of Engineering (Honours) in Software and Electronic Engineering

GMIT Department of Electrical and Electronic Engineering

Year 1

Industrial Automation

Gabriel Farragher 2022

# Student Details:

- Date: January – April 2022

- Module: Industrial Automation

- Student Name:

- Student Number:

- Lecturer: Gabriel Farragher

- Document: Lab Reports

# Table of Contents:

1	Student Details:	2

2	Table of Contents:	3

3	Table of Figures:	3

4	GX Works 2– Lab 4 Exercise 1 – FB Two Way SW:	5

4.1	Introduction to Function Blocks:	5

4.1.1	Step 1:	6

4.1.2	Step 2:	7

4.1.3	Step 3:	9

4.1.4	Step 4:	10

4.1.5	Step 5:	12

4.1.6	Step 6:	13

# Table of Figures:

No table of figures entries found.

Notes:

# GX Works 2– Lab 4 Exercise 1 – FB Two Way SW:

Lab Overview:

- In a previous exercise (Exercise 1) a Two-Way Switch was used in a School, connected as Inputs to a PLC and controlling the Stairs Light.

- The school has a requirement for many more of these Two-Way Switching configurations.

- To keep the PLC program lean, Function Blocks FB will be created.

## Introduction to Function Blocks:

- FB is an abbreviation for a Function Block that is designed to convert a ladder block, which is used repeatedly in a sequence program, into a component (FB) to be utilized in a sequence program.

- This not only increases the efficiency of program development but also reduces programming mistakes to improve program quality.

- In this Lab add one FB for the Stairs Two-Way SW and another for the Hall Two-Way SW.

- Develop the Ladder Logic for this operation.

- Include Comments.

PLC I/O Addresses:

- iHallway_SW1 		 	X1

- iHallway_SW2		 	X2

- oHallway_Light		 	Y1

- iStairs_SW1			 	X3

- iStairs_SW2		 		X4

- oStairs_Light	 			Y2

[Note: Save PLC software file as:  GX W2 L4 Ex 1 GF B]

### Step 1:

- Open GX Works 2

- Setup new project. [FXCPU, FX3G, Structured Project, Structured Ladder/FBD]

- Left click on FB/FUN

### Step 2:

- Right click and Add new.

- Enter fb_2_Way_SW as Data Name.

- Within the FB/FUN program create the following code:

- Note the bSW1 ‘Class’ is changed to VAR_INPUT

- Note the bSW2 ‘Class’ is changed to VAR_INPUT

- Note the bLight ‘Class’ is changed to VAR_OUTPUT

### Step 3:

- Check does your ‘Local Label’ within the FB/FUN match the following:

- Rebuild [Compile your code]:

- It is blue?

### Step 4:

- Now return to your POU_01 Program.

- Left click on the fb_2_Way_SW you have just created and drag into POU_01 program.

- Rename to Hallway_Control.

### Step 5:

- Go to Global Label and Open Global1.

- Enter the following:

### Step 6:

- Return to the POU_01 program and enter the following:

- Rebuild [Compile] and ensure no error.

- Run program to test.